# 6. ASSESSMENT GUIDELINES

These guidelines provide standardized criteria for assessing and reporting marine fouling, coating condition, and visible damage during underwater inspection. Extent and severity are visual estimates based on accessible areas and prevailing underwater visibility conditions.

# 6.1 FOULING CLASSIFICATION

Biofouling is classified from Rating 0 to 5 according to visible fouling growth and estimated surface coverage, based on IMO Annex 17 and the Visual Biofouling Rating Guides of NZ, AUS, and BRZ.

Biofouling Rating Scale – IMO Definition

| Rating | Classification | Surface Coverage | Definition / Description |
| --- | --- | --- | --- |
| 0 | Clean / No Fouling | 0% | No visible biofouling present on the hull surface. |
| 1 | Mico fouling | 1~100% / Slime Only | Slime or biofilm layer present. No visible macrofouling. |
| 2 | Light Macro fouling | 1~5% | Early visible macrofouling in limited or scattered areas. |
| 3 | Medium Macro fouling | 6~25% | Macrofouling forming distinct patches or clusters. |
| 4 | Heavy Macro fouling | 26~50% | Extensive macrofouling covering a broad area. |
| 5 | Severe Macro fouling | 51~100% | Severe or near-total macrofouling coverage. |

Coverage is estimated as a percentage of the total observable area for a specific component or hull section. Used for quantifying both fouling and coating breakdown. 0%; 5%; 10%; 20%; 30%; 40%. 50%; 60%; 70%; 80%; 90%; 100%.

Marine Environmental & Biosecurity Implications

Ratings 1 to 5 indicate progressively increasing marine environmental and biosecurity risk. Rating 1 generally requires standard monitoring, Rating 2 requires enhanced monitoring and possible pre-emptive cleaning, and Rating 3 indicates that cleaning or other intervention is recommended or may be required. Ratings 4 and Rating 5 represent critical to extreme risk levels, where immediate corrective action, containment cleaning, dry-docking, emergency offshore cleaning, or other approved control measures may be required depending on local regulations and the vessel’s next operational area.

6. ASSESSMENT GUIDELINES

# 6.2 OBSERVED CONDITION ASSESSMENT

This scale defines the attention grade for observed coating and visible condition findings, based on the level of review required.

Observed Rating Scale

| Rating | Classification / Level | Review Level | Definition / Description |
| --- | --- | --- | --- |
| 1 | Normal / Trace | No action required | No abnormal or only trace-level condition observed. No remarkable condition is identified. |
| 2 | Minor Observation | Record only | Minor localized or superficial condition observed. The condition is recorded for reference. |
| 3 | Notable Observation | Follow-up recommended | Clearly identifiable condition observed. Follow-up review is recommended depending on location, extent, and service relevance. |
| 4 | Significant Observation | Further review required | Significant condition observed over a broader area or at a function-related location. Further review or technical confirmation is required. |
| 5 | Critical Observation | Immediate review required | Critical condition observed requiring immediate review. Corrective action or operational decision may be required depending on vessel condition and relevant requirements. |

# 7. DETAILED SERVICE RECORD Photo-based evidence and condition records by work area This section provides photo-based records of the inspected and/or serviced areas, including work location, condition findings, work performed, and supporting visual evidence. For overall results and assessment criteria, refer to the preceding Overall Summary and Assessment Guidelines sections.